# TICELL® | Single blade louvre – mounted on T15/T24 carrier

Single blade open ceiling consisting of a substructure [T-shaped rail] and prefabricated panel elements.

The T-shaped rail is suspended rigidly from the mounting level [e.g. bare ceiling] via a suspension hanger for threaded rod with M6 threaded rods using technical approved dowels [distance of the suspensions according to EN 13964]. The distance between the T-profiles depends on the specified module dimension – according to the panel size length.

The blades of the single blade louvre type TICELL® have woven structure so that the use of a lighting from the ceiling void results in desired reflections or lighting effects. The panel elements are mounted at the module spacing and consist of square cells. The individual panels are self-adjusting. The individual panel elements are hung into the T-profiles [semi-concealed construction].

Typically, the edge connection is carried out by means of a shadow gap.

All parts are made of pre-coated or anodized aluminium.

| Pos. m² | Panels – TICELL® single blade open ceiling Panel size [LxW]:  600x600mm  1200x600mm Material thickness: 0.6mm Cell dimension:  33.33x33.33mm  54.55x54.55mm Height: 40mm Colour:  White  Black  Natural anodized Material: Aluminium Unit price: ............… €/m² Total price: ............… € |
| --- | --- |
| Pos. m² | T-shaped rail [T15/T24] Colour:  White Other colours on request Unit price: ............… €/m² Total price: ............… € |